# Advice on Assessment of Development Applications in the Vicinity of Gas and Liquid Petroleum Pipelines, Licensed under the Energy Resources Act 2000

Revision 1.0 – 14th July 2026

# Purpose

This document is intended to provide assurance to the Department of Energy and Mining (DEM) in its function as regulator under the Energy Resources Act 2000 (ERA) and a referral body under the Planning, Development and Infrastructure Act 2016 (PDI Act) that a certain proposed development is in line with the intent of the ERA and the Gas and Liquid Petroleum Pipelines or Gas and Liquid Petroleum Pipeline (Facilities) (Overlay) in the Planning and Design Code (Code).

This process has been implemented to encourage Applicants and Pipeline Licensee(s) to determine land use changes and potential arsing threats, to assess and document the risks introduced by developments and to document agreement to any relevant actions or variations to a development that are required to meet the Desired Outcome (DO) of an Overlay.

This process will facilitate timely assessment of Development Applications by the Regulation and Compliance Division (RCD) of DEM and will inform the basis of a direction to approve a development.

# Workflow

The checklists provided are to be completed by the appropriate representatives of the Applicant, approved by the relevant Pipeline Licensee and submitted to the Regulation and Compliance Division (RCD) of the Department for Energy and Mining (DEM). This checklist should be completed and provided, for any application subject to the Overlay(s), as part of the Development Application,

Provision of a completed copy of this checklist will expedite RCD’s assessment.

In general, RCD’s formal submissions will be prepared using the agreed responses provided in Table B and Table C. However, it is RCD’s responsibility to independently assess the information before finalising and responding formally. In the event of a discrepancy between RCD’s assessment and the agreements outlined between the Applicant and Pipeline Licensee, the RCD will consult the Applicant and Pipeline Licensee and seek to resolve any discrepancies prior to finalising their approval.

For further information regarding this document please contact:

Michael Malavazos

Director Energy Regulation

Regulation and Compliance Division

Department for Energy and Mining

All contact and correspondence should be electronically to RCD’s Engineering Inbox:

DEM.EnergyRegulation@sa.gov.au

Direct contact with Licensee can made via the following addresses -

Pipeline License Contacts

Epic Energy

PL(s) 1, 12, 3 and 4: Land.Management@epic.com.au

Santos

PL 2: Landholder.Contracts@santos.com

AGIG

PL 6 and 11: RCANetworks@agig.com.au

SEA Gas

PL 13:  development.application@seagas.com.au

APA

PL 7, 8, 16 and 18: planningsa@apa.com.au

Contents

1	Purpose	1

2	Workflow	1

3	Part B – Assessment Against ‘Deemed to Satisfy’ Criteria Met for Pipeline and Facility Overlays	5

3.1	Pipeline Operator Endorsement of Part B	7

3.2	Applicant Endorsement of Part B	7

4	Part C – protective measures	8

4.1.	Part C1	8

4.2.	Part C2	9

4.3.	Part C3	9

4.4.	Pipeline Operator Endorsement of Part C Conditions	10

4.5.	Proponent Endorsement of Part C Conditions	10

5	Recommendation to DEM	11

PART A Land Division Application Details (to be completed by Applicant and Licensee)

| Development application number (SAP) |  |
| --- | --- |
| Pipeline Licence |  |
| Location of development |  |
| Nature of development |  |
| Land use classification the pipeline is designed for in the location of the development |  |
| Proposed land use for the development |  |
| Contact details of proponent representative |  |
| Contact details of Pipeline Licensee representative |  |
| Attachments | Any relevant supporting documentation. e.g. Site layouts, development plans, service plans |

# Part B – Assessment Against ‘Deemed to Satisfy’ Criteria Met for Pipeline and Facility Overlays

| Performance Outcome |  | Deemed to Satisfy Criteria | DTS Criteria Not met |
| --- | --- | --- | --- |
| Gas and liquid Petroleum Pipelines Overlay |  |  |  |
| PO 1.1 | Community exposure to a potential hazard from the failure of a gas or liquid petroleum pipeline is mitigated by locating development that may accommodate or result in large congregations of people, buildings for housing and / or caring for vulnerable people and community facilities outside areas that pose an unacceptable risk to protect life. | Development satisfies one of the following: - it does not comprise: - child care facility - caravan and tourist park - educational facility - buildings comprising 3 or more building levels - land division creating allotments under 1ha for residential purposes (except where the existing allotment is less than 1ha) - prison - residential park - retirement facility - student accommodation - supported accommodation - shop or shops with a gross leasable floor area of 1000m2 or greater - tourist accommodation - stadium - a class of development referred to in part (a), or any combination thereof, which will occur in accordance with an agreement under section 123 of the Planning, Development and Infrastructure Act, 2016 | ☐ Go To C1 |
| PO1.2 | Emergency service and major community health related facilities are located outside areas where a gas or liquid petroleum pipeline failure may disrupt ongoing operations to maintain the response capacity in the event of an emergency. | Development comprises of any of the following: - Emergency services facility Hospital. | ☐ Go to C1 |
| PO1.3 | Development involving the manufacture, collection, handling or bulk storage of flammable, explosive, or otherwise hazardous materials is located and designed to avoid escalating the potential for and effects of a gas or liquid petroleum pipeline failure. | Development comprises any of the following: - General industry - Special industry - Landfill - Renewable energy facilities - Electricity substation - Fuel depot - Retail fuel outlet - Store - Warehouse - Waste treatment facility And the Development does not have an agreement under section 123 of the Planning, Development and Infrastructure Act, 2016. | ☐ Go to C2 |
| Gas and liquid Petroleum Pipelines (Facilities) Overlay |  |  |  |
| PO 1.1 | Development (including land division) does not present a risk to public health and safety due to any of the following: - continuous noise associated with pipeline facilities used for energy transportation that exceeds the Environment Protection (Commercial and Industrial Noise) Policy - potential for occasional noise associated with high pressure venting. | Development is within a facility overlay and (including land division creating additional allotments for such purposes) comprises other than following: - open space - roadways - a class of development that will occur in accordance with an agreement under section 123 of the Planning, Development and Infrastructure Act, 2016 - a dwelling or ancillary building/structure on allotment approved for residential purposes. | ☐ Go to C3 |

## Pipeline Operator Endorsement of Part B

Authorised Representatives Name:________________________________________________________________________________________________

Authorised Representatives Role:_________________________________________________________________________________________________

Authorised Representatives Signature: _____________________________________________________________________________________________

## Applicant Endorsement of Part B

Authorised Representatives Name:________________________________________________________________________________________________

Authorised Representatives Role:_________________________________________________________________________________________________

Authorised Representatives Signature: _____________________________________________________________________________________________

# Part C – protective measures

## Part C1

|  | C1 | Conditions (e.g.,Safety Management Study) | Risk Acceptable if Conditions are met |
| --- | --- | --- | --- |
| 1 | Describe the aspects of the development that do not meet the Deemed to Satisfy (DTS) criteria | Basis of Approval | ☐ |
| 2 | Are additional protection measures required to meet minimum physical or procedural requirements for land use? | List required protective measures | ☐ |
| 3 | Is additional protection required due to a high consequence area to demonstrate the risk of failure is ALARP? | List required protective measures | ☐ |
| 4 | Does the development introduce new threats to the pipeline that require additional mitigation? | List required protective measures | ☐ |

## Part C2

|  | C2 |  |  |
| --- | --- | --- | --- |
| 1 | List type and quantity of flammable, explosive, or otherwise hazardous materials | Basis of Approval |  |
| 2 | Are there design requirements to avoid escalating the potential for and effects of a gas or liquid petroleum pipeline failure? | List agreed design measures | ☐ |
| 3 | If the development includes electricity transmission infrastructure has an assessment been undertaken in accordance with AS 4853? | Attach report and list mitigation measures required | ☐ |
| 4 | Does the development introduce new threats to the pipeline that require additional mitigation? | List required protective measures | ☐ |

## Part C3

|  | C3 |  |  |
| --- | --- | --- | --- |
| 1 | Describe the development that is within the Gas and liquid Petroleum Pipelines (Facilities) Overlay | Basis of Approval |  |
| 2 | Can noise be mitigated to an acceptable level? |  |  |
| 3 | Have mitigation measures been agreed for noise? | List agreed mitigation measures | ☐ |

## Pipeline Operator Endorsement of Part C Conditions

Authorised Representatives Name:_________________________

Authorised Representatives Role:_________________________________________________________________________________________________

Authorised Representatives Signature: _____________________________________________________________________________________________

## Proponent Endorsement of Part C Conditions

Authorised Representatives Name:_________________________________________________________________________________________________

Authorised Representatives Role:_________________________________________________________________________________________________

Authorised Representatives Signature: _____________________________________________________________________________________________

# Recommendation to DEM

The Applicant and Pipeline Licensee of development application (Application #) have reached an agreed outcome of:

- No Impact – No further assessment required

- No Land Use Change - Additional Threats Controlled / AS2885 requirements satisfied

- Change in Land Use - Additional Threats  Controlled / AS2885 requirements satisfied

- Not Acceptable

As per the information declared in Table 2 and Table 3.

In regard to Desired Outcome in the Overlay (DO 1), being the management of risk to public safety, the environment and security of energy supply from the encroachment of development on strategic gas and liquid petroleum pipelines, and to Performance Outcomes PO 1.1 to 1.3 and their corresponding Designated Performance Features within the Overlay.

Also considering the Energy Resources Act 2000, Energy Resource Regulations 2013 (ER Regulations), and Australian Standard 2885: Pipelines – Gas and Liquid Petroleum (AS 2885), particularly as they relate to matters of safety (as referred to in DO 1) and the lawful continued operation of the Pipeline.

The Applicant and Pipeline Licensee  jointly recommend to DEM that the development proceeds as the controls outlined above will reduce the risk to ALARP.

I (Applicant representative name position), confirm that the above information is correct and reflects the understanding of (Applicant) and (Pipeline Licensee).

Applicant representative  -

Name: _____________________

Position:____________________

Signature: __________________

Date:     /   /

I (Pipeline Licensee representative name position), , confirm that the above information is correct and reflects the understanding of (Applicant) and (Pipeline Licensee).

Pipeline Licensee representative  -

Name: _____________________

Position: ____________________

Signature: __________________

Date:     /    /